# RZ 1508 CG-S

Pos. ...)  ... pcs.

Exit Luminaire acc. EN 60598-1, EN 60598-2-22 and EN 1838.

Exit luminaire Brilliant for ceiling mounting with rectangular section enclosure, 50 mm width, with easy relamping facility.

EVG with ENEC-test mark, certified from an independent testing lab, with integrated monitoring facility. Allows single luminaire monitoring with address-switches for 20 addresses.

An automatic switch off among the failures in the lamp circuit.

Switching modes (maintained/non-maintained and switched emergency luminaries) freely programmable with suitable emergency lighting systems. Mixed operation possible in a single circuit.

Marking: Silk-screened legend

Viewing distance: 28 m

- With single-sided legend panel

- With double-sided legend panel

- With crystal glass panel, engraved exit legend, not accd. to DIN/EN

Voltage range: 230 V 50/60 Hz, 220 V DC  +25/-20%

Current drawn (battery operation): 30 mA

Power consumption: 16 VA

Permissible temperature range: -10 °C up to +40 °C

Loop terminals: 3 x 2.5 sqmm

Insulation class: I, Degree of protection: IP 20

Inclusive lamp: 8 W T16, 450 Im

Dimensions of enclosure (mm): W = 338, H = 96, D = 75

Dimensions of panel (mm): W = 320, H = 190

Colour of enclosure:

- White, RAL 9010

Edge light for parallel to wall mounting

Type: RZ 1508 CG-S

Manufactured by: CEAG

# RZ 1608 CG-S

Pos. ...)  ... pcs.

Exit Luminaire acc. EN 60598-1, EN 60598-2-22 and EN 1838.

Exit luminaire Brilliant for ceiling mounting with rectangular section enclosure, 50 mm width, with easy relamping facility.

EVG with ENEC-test mark, certified from an independent testing lab, with integrated monitoring facility. Allows single luminaire monitoring with address-switches for 20 addresses.

An automatic switch off among the failures in the lamp circuit.

Switching modes (maintained/non-maintained and switched emergency luminaries) freely programmable with suitable emergency lighting systems. Mixed operation possible in a single circuit.

Marking: Silk-screened legend

Viewing distance: 28 m

- With single-sided legend panel

- With double-sided legend panel

- With crystal glass panel, engraved exit legend, not accd. to DIN/EN

Voltage range: 230 V 50/60 Hz, 220 V DC  +25/-20%

Current drawn (battery operation): 30 mA

Power consumption: 16 VA

Permissible temperature range: -10 °C up to +40 °C

Loop terminals: 3 x 2.5 sqmm

Insulation class: I, Degree of protection: IP 20

Inclusive lamp: 8 W T16, 450 Im

Dimensions of enclosure (mm): L = 430, W = 50, H = 96

Dimensions of panel (mm): W = 320, H = 190

Colour of enclosure:

- White, RAL 9010

Edge light for 90° wall mounting

Type: RZ 1608 CG-S

Manufactured by: CEAG

# RZ 1708 CG-S

Pos. ...)  ... pcs.

Exit Luminaire acc. EN 60598-1, EN 60598-2-22 and EN 1838.

Exit luminaire Brilliant for ceiling mounting with rectangular section enclosure, 50 mm width, with easy relamping facility.

EVG with ENEC-test mark, certified from an independent testing lab, with integrated monitoring facility. Allows single luminaire monitoring with address-switches for 20 addresses.

An automatic switch off among the failures in the lamp circuit.

Switching modes (maintained/non-maintained and switched emergency luminaries) freely programmable with suitable emergency lighting systems. Mixed operation possible in a single circuit.

Marking: Silk-screened legend

Viewing distance: 28 m

- With single-sided legend panel

- With double-sided legend panel

- With crystal glass panel, engraved exit legend, not accd. to DIN/EN

Voltage range: 230 V 50/60 Hz, 220 V DC  +25/-20%

Current drawn (battery operation): 30 mA

Power consumption: 16 VA

Permissible temperature range: -10 °C up to +40 °C

Loop terminals: 3 x 2.5 sqmm

Insulation class: I, Degree of protection: IP 20

Inclusive lamp: 8 W T16, 450 Im

Dimensions of enclosure (mm): W = 338, H = 96, D = 50

Dimensions of panel (mm): W = 320, H = 190

Colour of enclosure:

- White, RAL 9010

Ceiling mounting

Type: RZ 1708 CG-S

Manufactured by: CEAG

Pos. ...)  ... pcs.

Accessories:

- Chain fastening clip incl. quick mounting set

- Pendulum set 0.5m with wall bracket/termination box

Manufactured by: CEAG

# RZ 1808 CG-S

Pos. ...)  ... pcs.

Exit Luminaire acc. EN 60598-1, EN 60598-2-22 and EN 1838.

Exit luminaire Brilliant for recessed ceiling mounting, with easy relamping facility and ceiling bezel (plastics) in white colour.

EVG with ENEC-test mark, certified from an independent testing lab, with integrated monitoring facility. Allows single luminaire monitoring with address-switches for 20 addresses.

An automatic switch off among the failures in the lamp circuit.

Switching modes (maintained/non-maintained and switched emergency luminaries) freely programmable with suitable emergency lighting systems. Mixed operation possible in a single circuit.

Marking: Silk-screened legend

Viewing distance: 28 m

- With single-sided legend panel

- With double-sided legend panel

- With crystal glass panel, engraved exit legend, not accd. to DIN/EN

Voltage range: 230 V 50/60 Hz, 220 V DC  +25/-20%

Current drawn (battery operation): 30 mA

Power consumption: 16 VA

Permissible temperature range: -10 °C up to +40 °C

Loop terminals: 3 x 2.5 sqmm

Insulation class: I, Degree of protection: IP 20

Inclusive lamp: 8 W T16, 450 Im

Dimensions of panel (mm): W = 320, H = 190

Ceiling bezel (mm): L = 400, W = 100

Ceiling cut-out (mm): L = 387, W = 75

Min. mounting height (mm): 120

Type: RZ 1808 CG-S

Manufactured by: CEAG

Pos. ...)  ... pcs.

Accessories:

- Concrete mounting box with mounting set

Ceiling-ceiling bezel in:

- White, RAL 9010

Manufactured by: CEAG