29011 LED CG-S

Pos. ...)  ... pcs.

One-sided LED exit sign luminaire for wall mounting, according German / European standards EN 60598-1, DIN EN 60598-2-22, DIN 4844-1 and DIN EN 1838 for wall mounting; developed, manufactured and tested in keeping with ISO 9001.

Modular designed luminaire with one sided, rounded light exit surface, comprising: luminaire bootom and luminaire top made of 850°C glow wire resistant polycarbonate and single sided silk screened legend.

Light source: 3 x high efficient, white HighPower LED 1.1 W with a service life of 50.000 h on account of optimised LED operating conditions.

Special LED converter with integrated monitoring module for single luminaire monitoring

with 20-digit address switches.

Mixed operation of the connection systems (maintained light, non-maintained light and switched maintained light within a circuit without additional data or actuating cables to the luminaires is possible in combination with suitable group or central battery systems with STAR technology.

Viewing distance:											32 m

Luminous flux at the end of the rated service time:	100%

Housing material: 											Polycarbonate

Housing colour: 											Light grey, RAL 7035

Connection terminal:										clamp terminal 2.5 mm²

Supply voltage:												220 - 240 VAC, 50 Hz  /  176 - 275 VDC

Current consumption - battery operation:				19 mA

Power consumption including LED supply

(apparent power / effective power): 					7.6 VA / 4.4 W

Class of protection:										II

Type of protection											IP 41

Permissible ambient temperature: 						-10° Celsius to +40° Celsius

Dimensions (mm):											W = 342, H = 200, D = 68

Accessories:

..... pcs. Wire guard

Type: 29011 LED CG-S

Manufactured by: CEAG

29011 LED SL CG-S

Pos. ...)  ... pcs.

LED safety luminaire according German / European standards EN 60598-1, DIN EN 60598-2-22, DIN 4844-1 and DIN EN 1838 for wall mounting; developed, manufactured and tested in keeping with ISO 9001.

Modular designed luminaire with one sided, rounded light exit surface, comprising: luminaire bootom and luminaire top made of 850°C glow wire resistant polycarbonate and single sided silk screened legend.

Light source: 3 x high efficient, white HighPower LED 1.1 W with a service life of 50.000 h on account of optimised LED operating conditions.

Special LED converter with integrated monitoring module for single luminaire monitoring

with 20-digit address switches.

Mixed operation of the connection systems (maintained light, non-maintained light and switched maintained light within a circuit without additional data or actuating cables to the luminaires is possible in combination with suitable group or central battery systems with STAR technology.

Viewing distance:											32 m

Luminous flux at the end of the rated service time:	100%

Housing material: 											Polycarbonate

Housing colour: 											Light grey, RAL 7035

Connection terminal:										clamp terminal 2.5 mm²

Supply voltage:												220 - 240 VAC, 50 Hz  /  176 - 275 VDC

Current consumption - battery operation:				19 mA

Power consumption including LED supply

(apparent power / effective power): 					7.6 VA / 4.4 W

Class of protection:										II

Type of protection											IP 41

Permissible ambient temperature: 						-10° Celsius to +40° Celsius

Dimensions (mm):											W = 342, H = 200, D = 68

Accessories:

..... pcs. Wire guard

Type: 29011 LED SL CG-S

Manufactured by: CEAG

29021 LED CG-S

Pos. ...)  ... pcs.

One-sided LED exit sign luminaire for wall mounting, according German / European standards EN 60598-1, DIN EN 60598-2-22, DIN 4844-1 and DIN EN 1838 for wall mounting; developed, manufactured and tested in keeping with ISO 9001.

Modular designed luminaire with double sided, rounded light exit surface, comprising: of 850°C glow wire resistant polycarbonate and two sided silk screened legend.

Light source: 3 x high efficient, white HighPower LED 1.1 W with a service life of 50.000 h on account of optimised LED operating conditions.

Special LED converter with integrated monitoring module for single luminaire monitoring

with 20-digit address switches.

Mixed operation of the connection systems (maintained light, non-maintained light and switched maintained light within a circuit without additional data or actuating cables to the luminaires is possible in combination with suitable group or central battery systems with STAR technology.

Viewing distance:											32 m

Luminous flux at the end of the rated service time:	100%

Housing material: 											Polycarbonate

Housing colour: 											Light grey, RAL 7035

Connection terminal:										clamp terminal 2.5 mm²

Supply voltage:												220 - 240 VAC, 50 Hz  /  176 - 275 VDC

Current consumption - battery operation:				19 mA

Power consumption including LED supply

(apparent power / effective power): 					9.5 VA / 5.8 W

Class of protection:										II

Type of protection											IP 41

Permissible ambient temperature: 						-10° Celsius to +40° Celsius

Dimensions (mm):											W = 341, H = 202, D = 92

Accessories

..... pcs. ceiling fixing

..... pcs. Pendulum set 0.5 m

..... pcs. Wall bracket

..... pcs. chain fixing

Type: 29021 LED CG-S

Manufactured by: CEAG