# RZ 134 CG-S

Pos. ...)  ... pcs.

Emergency Luminaire acc. EN 60598-1, EN 60598-2-22 and EN 1838.

Enclosure made of aluminium section with plastic side caps, for trilaterally inscription with pre-assembled mounting set. For horizontal and vertical wall mounting and ceiling mounting.

EVG with ENEC-test mark, certified from an independent testing lab, with integrated monitoring facility. Allows single luminaire monitoring with address-switches for 20 addresses. An automatic switch off among the failures in the lamp circuit.

Switching modes (maintained/non-maintained and switched emergency luminaries) freely programmable with suitable emergency lighting systems. Mixed operation possible in a single circuit.

Marking: three-sided silk screened legend

Viewing distance: 12 m

Voltage range:

230V 50/60 Hz, 220 V DC +25/-20 %

Current drawn (battery operation): 20 mA

Power consumption: 10 VA

Permissible temperature range: -10 °C up to +40 °C

Loop terminals: 3 x 2.5 sqmm

Insulation class: I; Degree of protection: IP 30

Colour of enclosure: White

Inclusive lamp: 4 W T16

Dimensions (mm): L = 205, H = 75, D = 75

Type: RZ 134 CG-S

Manufactured by: CEAG

Pos. ...)  ... pcs.

Accessories:

- Wire guard

- Wall bracket

Manufactured by: CEAG