EVG 13.2 CG-S

High frequency electronic ballast in flame retardant polycarbonate enclosure to UL 94-VO. To be installed in pro-tection class I and II luminaires. For the supply and function monitoring of 4-pin fluorescent lamps 4 - 13 W. With ENEC-test mark, certified from an independent testing lab. Integrated monitoring station, allows single luminaire moni-toring with address-switches for 20 addresses. Automatic cutout for failures within the lamp circuit. Reverse battery protection. For connec-tion to CEAG emergency lighting system with 220V DC output and CEWA GUARD monitoring software. Luminous flux ratio FE in DC-supply approx. 75%.Switching modes (maintained/non-maintained and switched emergency luminaries) freely programmable with suitable emergency lighting systems. Mixed operation possible in a single circuit.

Permissible temperature range: -10°C up to +50°C

Voltage: 230 V 50/60 Hz, 220 V DC +25/-20 %

Loop terminals: 2.5 mm2

Dimensions (mm): H = 27.5; L = 140; W = 39

Degree of protection: IP 20

Accessories:

- Mounting plate with pull-relief

Dimensions (mm): L = 300, W = 50

Type: EVG 13.2 CG-S

Manufactured by: CEAG