N-EVG 1.. CG-S

EVG N-EVG ... CG-S High frequency electronic ballast in flame retardant polycarbo-nate enclosure. To be installed in protection class I and II luminaires. For 4-pin fluorescent lamps. With ENEC-test mark, certified from an independent testing lab. Integrated monitoring station, allows single luminaire monitoring with address-switches for 20 addresses. Automatic cutout for failures within the lamp circuit. Reverse battery protection. For connection to CEAG emergency lighting system with 220V DC output and CEWA GUARD monitoring software.

Switching modes (maintained/non-maintained and switched emergency luminaries) freely programmable with suitable emergency lighting systems. Mixed operation possible in a single circuit.

Luminous flux ratio in AC-supply 100%; the emergency light output is adjustable in 10% steps between 30% to 100% on each module.

Permissible temperature range: -10°C up to +50°C

Voltage: 230 V 50/60 Hz, 220 V DC +25/-20 %

Loop terminals: 2.5 mm2

Dimensions (mm): H = 30; L = 270; W = 30

Degree of protection: IP 20

Lamp:

- for 36 W/T26

Type: N-EVG 136 CG-S

- for 58 W/T26

Type: N-EVG 158 CG-S

- for 26 W/TC-TEL and TC-DEL

Type: N-EVG 126 CG-S

Manufactured by: CEAG